Examining issues related to PV generator components

Grasshope (hope.zh@gmail.com)

Aug 2018

- Original objectives

- Define shading surface as PV generator object in EnergyPlus through LB+HB

- get the following Building electricity usage and PV electricity production related output variables for data analysis:

Output:Variable,*,Generator Produced DC Electric Power, hourly; Output:Variable,*,Generator Produced DC Electric Energy, hourly; Output:Variable,*,Generator PV Array Efficiency, hourly; Output:Variable,*,Generator PV Cell Temperature, hourly; Output:Variable,*,Facility Total Surplus Electric Energy, hourly; Output:Variable,*,Facility Total Purchased Electric Energy, hourly; Output:Variable,*,Facility Total Produced Electric Energy, hourly; Output:Variable,*,Facility Net Purchased Electric Energy, hourly; Output:Variable,*,Facility Total Building Electric Demand Power, hourly; Output:Variable,*,Facility Total Building Electric Demand Energy, hourly; Output:Variable,*,Facility Total HVAC Electric Demand Power, hourly; Output:Variable,*,Facility Total HVAC Electric Demand Energy, hourly; Output:Variable,*,Facility Total Electric Demand Power, hourly; Output:Variable,*,Facility Total Electric Demand Energy, hourly; Output:Meter, Electricity:Building, hourly; Output:Meter, ElectricityNet:Facility, hourly; Output:Meter, ElectricityPurchased:Facility, hourly;

- Test model

- A simple model was created for the test which includes a small building with three thermal zones in different shapes and volumes. Three shading surfaces detached from the roofs were defined as PV surfaces.

- Default settings on building program and constructions are used.

- Chicago’s epw file was used to get both cooling and heating loads.

- Tests

Different combinations of the old and new versions of the following components were tested using the same building model. In total seven scenarios were tested.

| Component | Old version | New version |
| --- | --- | --- |
| Ladybug | V0066, JUN-02-2018 | V0066 JUL-11-2018 |
| Honeybee | V0063, JAN-20-2018 | V0063 JUN-29-2018 |
| PVgen | V0063, JAN-20-2018 | V0063, MAR-08-2018 |
| generationSystem | V0062, JUL-28-2017 | V0063, JAN-20-2018 |
| runEnergySimulation | V0063, JAN-20-2018 | v0063, JUL-24-2018 |
| exportToOpenStudio |  | V0063, AUG-08-2018 |
| EnergyPlus | V8.8.0 |  |

Compatibility between the components and main issues are summarized in the table below.

| Compatibility |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Ladybug V0066 JUN-02-2018 | Honeybee V0063 JAN-20-2018 | Ladybug V0066 JUL-11-2018 | Honeybee V0063 JUN-29-2018 |
| Old | PVgen V0063 JAN-20-2018 | Yes |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0062 JUL-28-2017 | Yes |  |  |  |
|  | runEnergySimulation V0063 JAN-20-2018 | Yes |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
| New | PVgen V0063 MAR-08-2018 | Error - Solution exception:__init__() takes exactly 12 arguments (8 given) |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0063 JAN-20-2018 | Yes |  | Yes |  |
|  | runEnergySimulation v0063 JUL-24-2018 | No (got reminder to update HB) |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
|  | exportToOpenStudio V0063 AUG-08-2018 |  |  | Yes |  |

- Results

- Combination A (Scenario 1) using the older version of the components generates the requested output variables correctly. (see result of Scenario 1 on page 5)

| Compatibility |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Ladybug V0066 JUN-02-2018 | Honeybee V0063 JAN-20-2018 | Ladybug V0066 JUL-11-2018 | Honeybee V0063 JUN-29-2018 |
| Old | PVgen V0063 JAN-20-2018 | Yes |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0062 JUL-28-2017 | Yes |  |  |  |
|  | runEnergySimulation V0063 JAN-20-2018 | Yes |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
| New | PVgen V0063 MAR-08-2018 | Error - Solution exception:__init__() takes exactly 12 arguments (8 given) |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0063 JAN-20-2018 | Yes |  | Yes |  |
|  | runEnergySimulation v0063 JUL-24-2018 | No (got reminder to update HB) |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
|  | exportToOpenStudio V0063 AUG-08-2018 |  |  | Yes |  |

- Combination B (Scenario 7) using the latest version of the components works only when the code of PVgen is revised in two places to suppress the error output of “-1”.

- A few of the key output variables requested were missing. (see result of Scenario 7 on page 13)

| Compatibility |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Ladybug V0066 JUN-02-2018 | Honeybee V0063 JAN-20-2018 | Ladybug V0066 JUL-11-2018 | Honeybee V0063 JUN-29-2018 |
| Old | PVgen V0063 JAN-20-2018 | Yes |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0062 JUL-28-2017 | Yes |  |  |  |
|  | runEnergySimulation V0063 JAN-20-2018 | Yes |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
| New | PVgen V0063 MAR-08-2018 | Error - Solution exception:__init__() takes exactly 12 arguments (8 given) |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0063 JAN-20-2018 | Yes |  | Yes |  |
|  | runEnergySimulation v0063 JUL-24-2018 | No (got reminder to update HB) |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
|  | exportToOpenStudio V0063 AUG-08-2018 |  |  | Yes |  |

Assuming the result of combination A is correct, the result on PV electricity yield for combination B is overestimated by 30.1%. (note the difference in energy unit)

Result of combination A                                                     Result of combination B

For combination B, electricity usage is about the same. But cooling loads is overestimated by 7.2%, and heating loads is slightly underestimated by only -0.3%.

Result of combination A                                                       Result of combination B

Without PV, the difference in electricity usage, cooling and heating loads between combination A and B is very small and could be regarded as zero.

The outcome of the seven scenarios tested are shown below.

Scenario 1 (S1) : test_PVgen_s1.gh

| Compatibility |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Ladybug V0066 JUN-02-2018 | Honeybee V0063 JAN-20-2018 | Ladybug V0066 JUL-11-2018 | Honeybee V0063 JUN-29-2018 |
| Old | PVgen V0063 JAN-20-2018 | Yes |  | Error The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0062 JUL-28-2017 | Yes |  |  |  |
|  | runEnergySimulation V0063 JAN-20-2018 | Yes |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
| New | PVgen V0063 MAR-08-2018 | Error - Solution exception:__init__() takes exactly 12 arguments (8 given) |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0063 JAN-20-2018 | Yes |  | Yes |  |
|  | runEnergySimulation v0063 JUL-24-2018 | No (got reminder to update HB) |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
|  | exportToOpenStudio V0063 AUG-08-2018 |  |  | Yes |  |

Results (correct):

End Uses (as reported in the html file generated)

|  | Electricity [kWh] | Natural Gas [kWh] | Additional Fuel [kWh] | District Cooling [kWh] | District Heating [kWh] | Water [m3] |
| --- | --- | --- | --- | --- | --- | --- |
| Heating | 0.00 | 0.00 | 0.00 | 0.00 | 17949.89 | 0.00 |
| Cooling | 0.00 | 0.00 | 0.00 | 7014.22 | 0.00 | 0.00 |
| Interior Lighting | 2588.20 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Exterior Lighting | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Interior Equipment | 1697.32 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| … | … | … | … | … | … | … |
| Total End Uses | 4285.52 | 0.00 | 0.00 | 7014.22 | 17949.89 | 0.00 |

Electric Loads Satisfied (as reported in the html file generated)

|  | Electricity [kWh] | Percent Electricity [%] |
| --- | --- | --- |
| Fuel-Fired Power Generation | 0.000 | 0.00 |
| High Temperature Geothermal* | 0.000 | 0.00 |
| Photovoltaic Power | 10531.273 | 245.74 |
| Wind Power | 0.000 | 0.00 |
| Power Conversion | -210.63 | -4.9 |
| Net Decrease in On-Site Storage | 0.000 | 0.00 |
| Total On-Site Electric Sources | 10320.647 | 240.83 |
| Electricity Coming From Utility | 2735.303 | 63.83 |
| Surplus Electricity Going To Utility | 8770.434 | 204.65 |
| Net Electricity From Utility | -6035.13 | -140.8 |
| Total On-Site and Utility Electric Sources | 4285.517 | 100.00 |
| Total Electricity End Uses | 4285.517 | 100.00 |

Scenario 2 (S2) : test_PVgen_s2.gh

generationSystem component is replaced with the latest version.

| Compatibility |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Ladybug V0066 JUN-02-2018 | Honeybee V0063 JAN-20-2018 | Ladybug V0066 JUL-11-2018 | Honeybee V0063 JUN-29-2018 |
| Old | PVgen V0063 JAN-20-2018 | Yes |  | Error The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0062 JUL-28-2017 | Yes |  |  |  |
|  | runEnergySimulation V0063 JAN-20-2018 | Yes |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
| New | PVgen V0063 MAR-08-2018 | Error - Solution exception:__init__() takes exactly 12 arguments (8 given) |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0063 JAN-20-2018 | Yes |  | Yes |  |
|  | runEnergySimulation v0063 JUL-24-2018 | No (got reminder to update HB) |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
|  | exportToOpenStudio V0063 AUG-08-2018 |  |  | Yes |  |

Results (correct):

Scenario 3 (S3) : test_PVgen_s3.gh

PVgen component is replaced with the latest version.

| Compatibility |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Ladybug V0066 JUN-02-2018 | Honeybee V0063 JAN-20-2018 | Ladybug V0066 JUL-11-2018 | Honeybee V0063 JUN-29-2018 |
| Old | PVgen V0063 JAN-20-2018 | Yes |  | Error The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0062 JUL-28-2017 | Yes |  |  |  |
|  | runEnergySimulation V0063 JAN-20-2018 | Yes |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
| New | PVgen V0063 MAR-08-2018 | Error - Solution exception:__init__() takes exactly 12 arguments (8 given) |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0063 JAN-20-2018 | Yes |  | Yes |  |
|  | runEnergySimulation v0063 JUL-24-2018 | No (got reminder to update HB) |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
|  | exportToOpenStudio V0063 AUG-08-2018 |  |  | Yes |  |

Error:

Got the following error even if the input data for each of the input nodes has been changed to a list of values:

1. Solution exception:__init__() takes exactly 12 arguments (8 given)

Unable to proceed to run the simulation.

Scenario 4 (S4) : test_PVgen_s4.gh

Old PVgen and generationSystem components were tested in new LB+HB

| Compatibility |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Ladybug V0066 JUN-02-2018 | Honeybee V0063 JAN-20-2018 | Ladybug V0066 JUL-11-2018 | Honeybee V0063 JUN-29-2018 |
| Old | PVgen V0063 JAN-20-2018 | Yes |  | Error The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0062 JUL-28-2017 | Yes |  |  |  |
|  | runEnergySimulation V0063 JAN-20-2018 | Yes |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
| New | PVgen V0063 MAR-08-2018 | Error - Solution exception:__init__() takes exactly 12 arguments (8 given) |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0063 JAN-20-2018 | Yes |  | Yes |  |
|  | runEnergySimulation v0063 JUL-24-2018 | No (got reminder to update HB) |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
|  | exportToOpenStudio V0063 AUG-08-2018 |  |  | Yes |  |

Error:

Got the following error on mismatching input lists, although all the inputs remain the same.

- The length of a list of inputs into either _name_, _cellsEfficiency, _integrationMode, _NoParallel,_Noseries, _costPVgen or _powerOutput is longer than the number of Honeybee surfaces connected to this component! e.g if you have 2 Honeybee surfaces you cannot have 3 values input into _costPVgen! Please check the inputs and try again!

Unable to proceed to run the simulation.

Scenario 5 (S5) : test_PVgen_s5.gh

New PVgen and generationSystem components were used.

| Compatibility |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Ladybug V0066 JUN-02-2018 | Honeybee V0063 JAN-20-2018 | Ladybug V0066 JUL-11-2018 | Honeybee V0063 JUN-29-2018 |
| Old | PVgen V0063 JAN-20-2018 | Yes |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0062 JUL-28-2017 | Yes |  |  |  |
|  | runEnergySimulation V0063 JAN-20-2018 | Yes |  |  |  |
| New | PVgen V0063 MAR-08-2018 | Error - Solution exception:__init__() takes exactly 12 arguments (8 given) |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! - PV_HBSurfaces output is “-1” |  |
|  | generationSystem V0063 JAN-20-2018 | Yes |  |  |  |
|  | runEnergySimulation v0063 JUL-24-2018 | No (got reminder to update HB) |  |  |  |
|  | exportToOpenStudio V0063 AUG-08-2018 |  |  |  |  |

Error:

- Still have the input list length mismatching issue

- PV_HBSurfaces output is “-1”, although the PV generator definition seems to be OK.

Scenario 6 (S6) : test_PVgen_s6.gh

The components used and the versions are the same as Scenario 5.

| Compatibility |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Ladybug V0066 JUN-02-2018 | Honeybee V0063 JAN-20-2018 | Ladybug V0066 JUL-11-2018 | Honeybee V0063 JUN-29-2018 |
| Old | PVgen V0063 JAN-20-2018 | Yes |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0062 JUL-28-2017 | Yes |  |  |  |
|  | runEnergySimulation V0063 JAN-20-2018 | Yes |  |  |  |
| New | PVgen V0063 MAR-08-2018 | Error - Solution exception:__init__() takes exactly 12 arguments (8 given) |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! - PV_HBSurfaces output is “-1” |  |
|  | generationSystem V0063 JAN-20-2018 | Yes |  |  |  |
|  | runEnergySimulation v0063 JUL-24-2018 | No (got reminder to update HB) |  |  |  |
|  | exportToOpenStudio V0063 AUG-08-2018 |  |  |  |  |

But line 443 and 458 of the code in PVgen component for the error return of “-1” were suppressed so that PV_HBSurfaces can output surfaces.

Error:

Although the three PV surfaces can be output from “PV_HBSurfaces” now, the following warning remains:

- The length of a list of inputs into either _name_, _cellsEfficiency, _integrationMode, _NoParallel,_Noseries, _costPVgen or _powerOutput is longer than the number of Honeybee surfaces connected to this component! e.g if you have 2 Honeybee surfaces you cannot have 3 values input into _costPVgen! Please check the inputs and try again!

The code revision above doesn’t work with the old and new versions of runEnergySimulation components, and the following error is reported:

Solution exception:'PV_gen' object has no attribute 'surfacename'

Scenario 7 (S7) : test_PVgen_s7.gh

Simulation was run by the exportToOpenStudio component

| Compatibility |  | Old |  | New |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Ladybug V0066 JUN-02-2018 | Honeybee V0063 JAN-20-2018 | Ladybug V0066 JUL-11-2018 | Honeybee V0063 JUN-29-2018 |
| Old | PVgen V0063 JAN-20-2018 | Yes |  | Error The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0062 JUL-28-2017 | Yes |  |  |  |
|  | runEnergySimulation V0063 JAN-20-2018 | Yes |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
| New | PVgen V0063 MAR-08-2018 | Error - Solution exception:__init__() takes exactly 12 arguments (8 given) |  | Error - The length of a list of inputs is longer than the number of Honeybee surfaces connected to this component! |  |
|  | generationSystem V0063 JAN-20-2018 | Yes |  | Yes |  |
|  | runEnergySimulation v0063 JUL-24-2018 | No (got reminder to update HB) |  | Error - Solution exception:'PV_gen' object has no attribute 'surfacename' |  |
|  | exportToOpenStudio V0063 AUG-08-2018 |  |  | Yes |  |

Simulation works

However, the PV electricity generation related output is not the same as the one generated using old versions of LB, HB, PVgen, generationSystem  and runEnergySimulation components.

Result of Scenario 7 (in GJ)

Result of Scenario 7 (in kWh)	                                                                  Result of S1 (in kWh)

Four output variables were also missing which were generated correctly in Scenario 1.

|  | Electricity load and generation related output variables requested |  |  | Scenario |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | Electric Load Center Produced Electric Energy |  |  |  |  |  |  |  |  | X |
| 2 | Electricity:Building |  |  |  |  |  |  |  |  | X |
| 3 | ElectricityNet:Facility |  |  |  |  |  |  |  |  | X |
| 4 | ElectricityPurchased:Facility |  |  |  |  |  |  |  |  | X |
| 5 | Facility Net Purchased Electric Energy |  |  |  |  |  |  |  |  |  |
| 6 | Facility Total Building Electric Demand Power |  |  |  |  |  |  |  |  |  |
| 7 | Facility Total Electric Demand Power |  |  |  |  |  |  |  |  |  |
| 8 | Facility Total HVAC Electric Demand Power |  |  |  |  |  |  |  |  |  |
| 9 | Facility Total Produced Electric Energy |  |  |  |  |  |  |  |  |  |
| 10 | Facility Total Purchased Electric Energy |  |  |  |  |  |  |  |  |  |
| 11 | Facility Total Surplus Electric Energy |  |  |  |  |  |  |  |  |  |
| 12 | Generator PV Array Efficiency |  |  |  |  |  |  |  |  |  |
| 13 | Generator PV Cell Temperature |  |  | X | X |  |  |  |  | X |
| 14 | Generator Produced DC Electric Energy |  |  |  |  |  |  |  |  |  |
| 15 | Generator Produced DC Electric Power |  |  |  |  |  |  |  |  |  |
|  | Generated | Not generated | X : missing |  |  |  |  |  |  |  |